# Supplementary Material

Threshold Robustness of Slip-Stick Peak Detection

This supplementary material supports the manuscript "Slip-Stick Dynamics in Butyl Pressure-Sensitive Adhesive-Silicone Release Liner Systems: A New Approach to Analyzing the Performance of Release Liners". It documents the noise floor, threshold-sensitivity check and exploratory configuration-level statistics for the slip-stick peak-count descriptor.

# S1. Dataset Scope

The analysis used the manuscript-relevant force-displacement datasets from the slip_stick repository. The silphan datasets were excluded because they are not part of the manuscript matrix. One older repeated C1E/Rossella/internal file and one file explicitly marked as a duplicate were also excluded. The final analysis included 42 liner-sealant-side configurations and 424 replicate traces.

For every trace, the same processing settings as in the manuscript were used: normalization to cN/25 mm, analysis over the 50-200 mm displacement window, a long Savitzky-Golay baseline, and peak detection on the residual force signal. The default threshold was 1.4 cN/25 mm.

# S2. Instrumental Noise and Threshold Margin

Table S1. Baseline noise summary from the initial near-zero-force segment of each raw trace.

| Metric | Value |
| --- | --- |
| Number of replicate traces | 424 |
| Number of liner-sealant-side configurations | 42 |
| Median baseline noise SD | 0.066 cN/25 mm |
| 95th percentile baseline noise SD | 0.116 cN/25 mm |
| Maximum baseline noise SD | 0.343 cN/25 mm |
| Median baseline maximum absolute noise | 0.166 cN/25 mm |
| 95th percentile baseline maximum absolute noise | 0.318 cN/25 mm |
| Maximum baseline maximum absolute noise | 0.933 cN/25 mm |
| Threshold / median noise SD | 21.3x |
| Threshold / 95th percentile noise SD | 12.0x |

The default 1.4 cN/25 mm threshold was therefore well above the measured noise level. It was about 21 times the median baseline noise SD and about 12 times the 95th percentile baseline noise SD.

# S3. Threshold Sensitivity

Table S2. Sensitivity of peak counts to the detection threshold. Spearman rho was computed across configuration-level mean peak counts relative to the default 1.4 cN/25 mm threshold.

| Threshold [cN/25 mm] | Spearman rho vs 1.4 | p value | Total peaks | Configs mean >= 1 peak | Configs mean >= 5 peaks |
| --- | --- | --- | --- | --- | --- |
| 0.5 | 0.821 | <0.001 | 10269 | 40 | 34 |
| 1.0 | 0.955 | <0.001 | 3879 | 34 | 18 |
| 1.4 | 1.000 | <0.001 | 2334 | 26 | 14 |
| 2.0 | 0.965 | <0.001 | 1486 | 20 | 8 |
| 3.0 | 0.880 | <0.001 | 1010 | 11 | 4 |

The absolute number of detected peaks decreased as the threshold increased, as expected. The ranking of configurations remained stable in the practical threshold range around the selected value: rho = 0.955 for 1.0 cN/25 mm and rho = 0.965 for 2.0 cN/25 mm, both relative to the 1.4 cN/25 mm analysis.

# S4. Configurations with the Highest Peak Counts

Table S3. Ten configurations with the highest mean peak counts at the default 1.4 cN/25 mm threshold. Values are means per replicate unless otherwise stated.

| Sealant | Liner | Side | n | Mean peaks 1.0 | Mean peaks 1.4 | Mean peaks 2.0 | Mean peaks 3.0 | Mean force [cN/25 mm] | Median noise SD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U2E | Rossella | outer | 10 | 42.5 | 42.3 | 42.0 | 41.5 | 238.4 | 0.059 |
| U2E | Crosil 42 | outer | 10 | 48.6 | 37.3 | 28.5 | 19.6 | 17.8 | 0.075 |
| T1EN | Rossella | outer | 10 | 40.5 | 24.1 | 12.7 | 5.7 | 25.9 | 0.056 |
| T2E | Dolpap | inner | 10 | 25.5 | 16.2 | 8.6 | 3.5 | 7.0 | 0.065 |
| T1E | Rossella | inner | 10 | 27.8 | 11.7 | 6.1 | 2.5 | 9.0 | 0.080 |
| C1EN | Rossella | inner | 10 | 14.9 | 10.9 | 8.5 | 6.5 | 15.0 | 0.059 |
| T2EN | Rossella | inner | 10 | 12.6 | 9.3 | 6.7 | 4.5 | 12.9 | 0.071 |
| T2E | Rossella | inner | 10 | 11.6 | 8.7 | 5.8 | 2.0 | 10.8 | 0.066 |
| C1E | Rossella | outer | 12 | 23.6 | 7.2 | 1.2 | 0.333 | 12.5 | 0.080 |
| T2EN | Crosil 42 | outer | 10 | 15.7 | 7.0 | 1.8 | 0.600 | 8.3 | 0.072 |

The Rossella/U2E/outer configuration remained extreme at every threshold tested. Other configurations changed in absolute peak count with threshold, but the same high-instability group remained visible across the 1.0-2.0 cN/25 mm range. High peak count should not be read as identical to high mean release force; it describes instability of the force trace after baseline correction.

# S5. Exploratory Configuration-Level Statistics

Table S4. Exploratory paired tests for the configuration-level peak-count descriptor at the default 1.4 cN/25 mm threshold.

| Effect tested | Test | Blocks | Statistic | p value |
| --- | --- | --- | --- | --- |
| liner paired within sealant-side blocks | Friedman | 14 | 3.857 | 0.145 |
| sealant paired within liner-side blocks | Friedman | 6 | 4.599 | 0.596 |
| side paired within sealant-liner blocks | Wilcoxon signed-rank | 21 | 99.500 | 0.837 |

No single main factor by itself explained the peak-count descriptor at the configuration-mean level. This supports the interpretation used in the manuscript: slip-stick tendency is governed by the specific sealant-liner-side configuration rather than by liner type, sealant type or liner side alone.

# S6. Machine-Readable Supplementary Data

data/threshold_robustness.csv: threshold-sensitivity summary.

data/top_peak_configs.csv: ranked configuration summary by default-threshold peak count.

data/paired_configuration_tests.csv: exploratory paired tests on configuration-level means.

data/configuration_summary.csv: configuration-level release-force, noise and peak-count summaries.

data/replicate_level.csv: replicate-level processed metrics.

data/summary.json: machine-readable summary of the analysis run.