# Supplementary Methodological Background

## Threshold Robustness of Slip-Stick Peak Detection

This supplementary note supports the operational residual-force threshold of 1.4 cN/25 mm used for slip-stick peak detection. The threshold is not treated as a universal material constant; it is a reproducible detection criterion applied to all traces in the manuscript matrix.

The values below are generated by `python scripts/generate_publication_outputs.py` from the publication dataset manifest.

### S1. Dataset Scope

Replicate traces: 424

Liner-sealant-side configurations: 42

Analysis window: 50-200 mm displacement.

Force normalization: cN/25 mm from the 90 mm collection width.

Peak definition: one contiguous positive residual excursion above the threshold is counted as one event.

### S2. Noise Margin

| Metric | Value |
| --- | --- |
| Median baseline noise SD | 0.066 cN/25 mm |
| 95th percentile baseline noise SD | 0.116 cN/25 mm |
| Maximum baseline noise SD | 0.343 cN/25 mm |
| Threshold / median noise SD | 21.3x |
| Threshold / 95th percentile noise SD | 12.0x |

The 1.4 cN/25 mm threshold is therefore well above the measured baseline noise floor.

### S3. Threshold Sensitivity

Peak counts were recalculated at nearby residual-force thresholds. Spearman rho was computed across configuration-level mean peak counts relative to the default 1.4 cN/25 mm threshold.

| Threshold [cN/25 mm] | Spearman rho vs default | p value | Total peaks | Configs mean >= 1 peak | Configs mean >= 5 peaks |
| --- | --- | --- | --- | --- | --- |
| 0.5 | 0.827 | <0.001 | 3450 | 37 | 25 |
| 1.0 | 0.968 | <0.001 | 1427 | 27 | 9 |
| 1.4 | 1.000 | reference | 904 | 22 | 6 |
| 2.0 | 0.954 | <0.001 | 553 | 15 | 3 |
| 3.0 | 0.870 | <0.001 | 343 | 8 | 1 |

The absolute number of detected peaks decreases as the threshold increases, as expected. The configuration-level rankings remained strongly correlated with the default analysis at 1.0 cN/25 mm (rho = 0.968) and 2.0 cN/25 mm (rho = 0.954).

This supports using 1.4 cN/25 mm as an operational threshold rather than indicating that the slip-stick ranking depends on a single arbitrary value.

### S4. Highest Peak-Count Configurations

| Sealant | Liner | Side | n | Mean peaks 1.0 | Mean peaks 1.4 | Mean peaks 2.0 | Mean force [cN/25 mm] | Median noise SD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U2E | Rossella | outer | 10 | 12.4 | 12.3 | 12.1 | 238.4 | 0.059 |
| U2E | Crosil 42 | outer | 10 | 13.1 | 9.8 | 7.8 | 17.8 | 0.075 |
| T1EN | Rossella | outer | 10 | 11.6 | 7.7 | 5.4 | 25.9 | 0.056 |
| T1EN | Crosil 42 | inner | 10 | 8.4 | 5.7 | 2.8 | 10.8 | 0.069 |
| C1E | Rossella | inner | 5 | 12.6 | 5.6 | 1.0 | 10.5 | 0.039 |
| T1E | Rossella | inner | 10 | 8.4 | 5.6 | 4.0 | 9.0 | 0.080 |
| C1E | Rossella | outer | 10 | 12.3 | 4.3 | 1.2 | 12.5 | 0.080 |
| T2EN | Crosil 42 | outer | 10 | 6.8 | 3.9 | 1.6 | 8.3 | 0.072 |
| T2E | Dolpap | inner | 10 | 4.6 | 3.7 | 2.1 | 7.0 | 0.065 |
| U2E | Dolpap | outer | 10 | 4.2 | 3.2 | 2.3 | 8.4 | 0.058 |

High peak count should not be read as identical to high mean release force. It describes instability of the residual force trace after baseline correction.

### S5. Machine-Readable Data

publication/generated/data/threshold_noise_summary.csv

publication/generated/data/threshold_robustness.csv

publication/generated/data/top_peak_configs.csv

publication/generated/data/threshold_robustness_summary.json

publication/generated/data/configuration_summary.csv

publication/generated/data/replicate_metrics.csv